# A Sparse Bayesian Learning Approach to Multi-Component Jitter Decomposition

Abstract

Accurate decomposition of individual jitter components from a time interval error (TIE) sequence is essential for signal integrity assessment in high-speed digital communications. Existing frequency-domain methods estimate the periodic jitter (PJ) frequency by FFT peak-picking and then fit the amplitudes by least squares; however, the non-uniform edge spacing of pseudo-random binary sequence (PRBS) signals violates the uniform-sampling assumption underlying the FFT, producing a systematic bias in the estimated frequency that propagates as amplitude errors to the PJ component and beyond.

To address this limitation, a sparse Bayesian learning (SBL) approach to jitter decomposition is proposed. A redundant dictionary whose frequency resolution can be designed independently is constructed, and its active columns are identified by the automatic relevance determination (ARD) mechanism in place of FFT peak-picking. To accelerate the inference, the active frequencies are first localised on a coarse grid by the fast relevance vector machine (FastRVM); a fine ARD inference is then carried out within the resulting neighbourhoods.

Experimental results show that, relative to the above baseline, the proposed method confines the PJ estimation error within 0.5 ps over the 100 kHz – 15 MHz band and effectively suppresses error propagation to the ISI and RJ components, validating both its accuracy and its stability.

Keywords: jitter decomposition; periodic jitter; sparse Bayesian learning; redundant dictionary; signal integrity

1 Introduction

As data rates in high-speed digital communications continue to rise, systems impose ever more stringent constraints on timing precision and signal integrity. Jitter, being the dominant factor that affects the bit error rate (BER), has become a key indicator in signal integrity assessment [1]. Acquiring the time interval error (TIE) sequence with an oscilloscope and decomposing it precisely into its constituent components is the foundation for evaluating system performance and locating sources of disturbance [2]. According to physical origin and statistical properties, total jitter (TJ) is conventionally split into random jitter (RJ) and deterministic jitter (DJ); the latter is further subdivided into inter-symbol interference (ISI), duty-cycle distortion (DCD) and periodic jitter (PJ) [3].

Existing jitter decomposition methods can be broadly grouped into statistical-domain, time-domain and frequency-domain approaches. Statistical-domain methods, exemplified by tail fitting [4] and the dual-Dirac model [5, 6], achieve a fast separation of RJ and DJ but provide no explicit model for the pattern dependence of ISI. Time-domain methods such as time lag correlation (TLC) [7] employ correlation functions to separate correlated and uncorrelated jitter components but cannot directly extract the frequency information of PJ. Frequency-domain methods such as the FFT-based spectral peak-picking [8] reveal the frequency structure of PJ intuitively but likewise lack an explicit model for the pattern dependence of ISI. Recently, deep-learning-based methods [9] have also made some progress, although they typically rely on large amounts of labelled data and offer limited interpretability.

Duan et al. [10] proposed a least-squares decomposition based on a linear superposition model of the TIE, in which the PJ frequency is located by FFT peak-picking and the amplitudes are then fitted by least squares (referred to here as LS-FFT). The method incorporates the four components — PJ, ISI, DCD and RJ — into a unified linear model, and the amplitude of each component can be estimated jointly from a small number of TIE samples. The accuracy of this method, however, hinges on an accurate FFT frequency estimate. The TIE is defined only at data transition edges; consecutive identical bits in real data patterns produce no transition, so adjacent TIE samples are not equally spaced on the time axis and the uniform-sampling assumption of the FFT is violated. The FFT peak therefore deviates from the true PJ frequency (Fig. 1). Furthermore, when the PJ frequency falls between two DFT bins, the picket-fence effect aggravates this bias. Fitting the PJ component with a frequency that has been shifted, the basis functions no longer align with the true signal; the amplitude estimate becomes inaccurate and errors propagate to the ISI and RJ components.

![[fig1_nonuniform_sampling.png]]

Fig. 1 Non-uniform edge distribution of a PRBS signal.

To address this issue, a multi-component jitter decomposition method based on sparse Bayesian learning (SBL) is proposed. Building upon the linear superposition model of Duan et al. [10], a redundant dictionary containing a large set of candidate frequencies is constructed using the actual transition instants in place of a single frequency column, so that the spacing and number of candidate frequencies are no longer constrained by the number of samples. The automatic relevance determination (ARD) mechanism inherent in SBL is then used to select the true PJ frequency directly from the redundant dictionary. FFT peak-picking is no longer required, and the frequency-estimation bias caused by non-uniform sampling and the picket-fence effect is avoided.

2 Signal Model and Dictionary Construction

2.1 Linear TIE Model

The TIE at the kth transition edge can be decomposed, according to its physical origin, into a linear superposition of deterministic components and random noise:

y(k)=yPJ(k)+yISI(k)+yDCD(k)+v(k)

where v(k) comprises random jitter and observation noise. Stacking the TIE samples over N transition edges in matrix form [10]:

y=[A(f0)∣B∣C]x+v

The PJ sub-matrix A(f0)∈RN×2 has columns dncos(2πf0n/fs) and dnsin(2πf0n/fs), where dn is the transition indicator (1 at a transition, 0 otherwise); the DCD sub-matrix B∈RN×1 is set to +1 on rising edges and −1 on falling edges; the ISI sub-matrix C∈RN×2L buckets the rows by the history pattern within memory depth L, placing a 1 in the column corresponding to the current pattern of each row.

The accuracy of the solution to (2) hinges on whether A(f0) reflects the true phase structure of the PJ component on the TIE sequence. The phase term 2πf0n/fs in the nth row of A(f0) approximates the nth transition as a uniformly sampled point at n/fs. Under PRBS patterns, however, consecutive identical bits produce no transition, so adjacent transitions are not equally spaced in time; the phase bias introduced by this approximation causes the FFT-based estimate of f0 to deviate systematically from its true value, and the bias is propagated through the least-squares fit to the PJ and the other component amplitudes.

2.2 Redundant Frequency Dictionary

Rather than relying on FFT peak-picking, this paper places K candidate frequencies in the target band, constructs the dictionary column for each candidate using the actual transition arrival times tn, and identifies the active frequencies by SBL. Here tn denotes the ideal arrival time of the nth transition edge: in a PRBS test the transmitted pattern is known, so the transition positions can be determined directly from the pattern, tn=edge_index(n)/fs, and this information is available when the TIE sequence is extracted.

For each candidate frequency fi, a pair of cosine and sine basis columns is constructed:

hc,i&=[d1cos(2πfit1),…,dNcos(2πfitN)]Ths,i&=[d1sin(2πfit1),…,dNsin(2πfitN)]T

A PJ component with arbitrary initial phase can be expressed linearly as aihc,i+bihs,i, converting non-linear phase estimation into linear coefficient estimation. Concatenating the basis-column pairs of the K candidate frequencies horizontally yields the PJ sub-dictionary

HPJ=[hc,1,hs,1,…,hc,K,hs,K]∈RN×2K

The spacing and number of the candidate frequencies f1,…,fK are not constrained by the DFT grid Δf=fs/N and may be chosen flexibly according to the target band. Concatenating HPJ with B and C yields the global redundant dictionary:

$$\mathbf{H} = [\mathbf{H}_{PJ} \mid \mathbf{B} \mid \mathbf{C}] \in \mathbb{R}^{N \times M}, \quad M = 2K + 1 + 2^L \tag{5}$$

After solving for the coefficient vector x, the physical quantity of each component is extracted as follows. The coefficient pair (ai,bi) at an active frequency fi gives the peak-to-peak PJ amplitude

Pk-PkPJ=2ai2+bi2

The DCD and ISI peak-to-peak values are Pk-PkDCD=2|xDCD| and Pk-PkISI=max(xISI)−min(xISI), respectively; the RMS RJ is estimated from the fitting residual,

σRJ=∥y−Hx∥22N−P

where P is the number of active columns in H.

Adjacent candidate-frequency columns of H are highly correlated, HTH is severely ill-conditioned, and a direct least-squares solution is numerically unstable. In a real system the number of PJ disturbance sources present simultaneously is small, so only a few entries of the coefficient vector are non-zero, and the problem reduces to identifying the columns associated with these non-zero coefficients and estimating their values. Sparse Bayesian learning (SBL) provides a probabilistic inference framework for sparse-recovery problems of this kind [11].

3 SBL-Based Jitter Decomposition Algorithm

The frequency resolution of the redundant dictionary is not bounded by the number of samples, but a finer resolution implies a larger number of candidate columns. Were a fine dictionary with thousands of columns to be constructed directly, ARD would have to update all columns simultaneously, with a computational cost that grows quadratically in the number of columns and prohibitive single-pass inference times. FastRVM, by contrast, performs a sequential update that processes only one column at each step and is efficient on large coarse-grid dictionaries; the coarse spacing, however, limits the achievable frequency resolution. The inference is therefore carried out in two stages: FastRVM is first used on a coarse grid to localise the approximate intervals of the PJ frequencies, and a fine dictionary is then constructed within these intervals so that ARD can deliver an accurate frequency and amplitude estimate. Both stages identify the active frequency columns by maximising the marginal likelihood; the complete algorithmic flow is shown in Fig. 2.

![[图表/fig3_flowchart.png]]

Fig. 2 Flowchart of the SBL-based jitter decomposition algorithm.

3.1 FastRVM Coarse Localisation

The aim of the coarse-localisation stage is to determine the approximate intervals of the PJ frequencies within the 100 kHz – 15 MHz band.

SBL assigns an independent zero-mean Gaussian prior to each entry of the coefficient vector w [11]:

wi∼N(0,αi−1)

where αi is the precision hyperparameter; αi→∞ effectively prunes the corresponding column from the model. The hyperparameters {αi} are estimated from the data by maximisation of the marginal likelihood, and on convergence only a few columns retain a finite αi, achieving sparse recovery.

100 candidate frequencies are uniformly placed across 100 kHz – 15 MHz, giving a 200-column PJ sub-dictionary by (3)–(4); concatenation with B and C yields a coarse global dictionary of 233 columns. Sequential sparse inference [12] is then applied to this dictionary by FastRVM. Let the observation model be y=Hw+ε with ε∼N(0,σ2I).

Rather than iterating on all columns simultaneously, FastRVM maintains an active set A, initially empty, and at each step decides whether to add or remove a single column. When the ith column is examined, its quality factor Qi and sparsity factor Si relative to the current residual are computed:

$$Q_i = \mathbf{h}_i^T \boldsymbol{\Phi}^{-1} \mathbf{y}, \quad S_i = \mathbf{h}_i^T \boldsymbol{\Phi}^{-1} \mathbf{h}_i \tag{9}$$

where hi is the ith dictionary column and Φ=σ2I+HAΛA−1HAT is the marginal covariance for the current active set A. Qi measures the contribution of column i to fitting the residual and Si reflects its energy. Together, they form the sparsity–quality criterion [12]:

$$\theta_i = Q_i^2 - S_i \tag{10}$$

If θi>0, the fit improvement obtained by including the column outweighs the complexity penalty of admitting it; the column is added to the active set, and αi is updated to its closed-form maximiser of the marginal likelihood:

αinew=Si2θi

If θi≤0, αi is set to infinity and the column is removed from the model. The above procedure is applied sequentially to each column and iterated until the active set no longer changes. After convergence, for every candidate frequency fk the relevances of its sine and cosine columns are summed, 1/α2k−1+1/α2k, and the candidates are ranked in decreasing order of this sum; the three highest are taken as the candidate centre frequencies f1,f2,f3 for the subsequent fine inference. The coarse-localisation result is shown in Fig. 3(a).

![[图表/fig2_fastrvm_compression.png]]

Fig. 3 FastRVM coarse localisation and ARD fine-dictionary construction.

3.2 ARD-Based Fine Frequency Identification

The frequency resolution of FastRVM on the coarse grid is approximately 150 kHz, so f1,f2,f3 provide only the approximate location of the true frequencies; a fine dictionary is required in the surrounding neighbourhoods to deliver an accurate frequency and amplitude estimate. A fine window of ±200 kHz is centred at each fi, covering the coarse-localisation error of FastRVM (about ±75 kHz) and ensuring that the true PJ frequency lies inside the window. 100 candidate frequencies are uniformly placed in each window with a spacing of approximately 4 kHz, and the three windows are merged to give 300 fine candidate frequencies. The fine global dictionary H∈RN×M is then assembled via (3)–(5), giving M=600+1+32=633 (Fig. 3(b)).

Adjacent fine-candidate columns are highly correlated and a direct least-squares solution is numerically unstable. The same ARD prior as in (8) is imposed on H. By the conjugacy of the Gaussian prior and the Gaussian observation model, the posterior distribution p(w∣y) is Gaussian, with mean and covariance

μ=σ−2ΣHTy, Σ=σ−2HTH+Λ−1

where Λ=diag(α1,…,αM). The posterior mean μ serves as the coefficient estimate.

The hyperparameters {αi} and the noise variance σ2 are not specified manually; they are estimated jointly from the data by maximising the marginal likelihood p(y∣{αi},σ2), with the update rules

αinew=1−αiΣiiμi2, (σ2)new=∥y−Hμ∥2N−i​(1−αiΣii)

where Σii is the ith diagonal entry of the posterior covariance matrix. The updates of αi and σ2 are alternated until convergence.

After inference, the posterior mean μ is taken directly as the coefficient estimate of each component: the coefficient pairs (ai,bi) at the active frequencies provide the peak-to-peak PJ amplitude via (6); the DCD and ISI coefficients give the corresponding component amplitudes; and the RMS RJ is estimated from the residual via (7).

4 Experimental Validation

To verify the effectiveness of the proposed method, simulation experiments are designed to evaluate the wide-band PJ estimation accuracy and the multi-component joint decomposition accuracy, comparing LS-FFT [10] against the proposed SBL method. The simulated data are TIE sequences generated under PRBS-7 patterns at a 2 Gbps data rate, with 50 000 samples collected, channel memory depth L=5, and DCD and ISI ground truths set to 1.0 ps and 0.16 ps, respectively. Since the FFT peak shift varies with the PJ frequency, the experiments scan the PJ frequency over 100 kHz – 15 MHz to compare the decomposition performance of the two methods. Two parameter sets are used: Sample 1 with PJ = 10.0 ps and RJ = 5.0 ps; Sample 2 with PJ = 6.0 ps and RJ = 5.0 ps.

Six frequency points (0.1, 0.5, 1, 5, 10 and 15 MHz) are selected within 100 kHz – 15 MHz, and the peak-to-peak PJ estimation errors of the two parameter sets are shown in Fig. 4.

![[图表/fig4_freq_accuracy_combined.png]]

Fig. 4 Wide-band PJ estimation errors for the two samples.

As shown in Fig. 4, in both samples and at all six frequency points the SBL PJ error remains below 0.5 ps with no clear dependence on frequency, whereas the LS-FFT PJ error grows with frequency and reaches 9.40 ps (Sample 1) and 5.54 ps (Sample 2) at 15 MHz, exceeding the SBL error by one to two orders of magnitude. This is because the LS-FFT PJ error is dominated by the FFT frequency-estimation bias: the higher the frequency, the larger the relative shift between adjacent DFT bins, and the larger the resulting estimation error. SBL identifies the active frequencies directly from the redundant dictionary and does not rely on the uniform-sampling assumption, so its error does not vary with frequency.

Three frequency points (0.1, 1.0 and 10.0 MHz) are then chosen to compare the simultaneous decomposition accuracy of the four components — PJ, DCD, ISI and RJ — and the errors are reported in Table 1.

Table 1 Multi-component decomposition errors (ps). Ground truth — Sample 1: PJ = 10.00, DCD = 1.00, ISI = 0.16, RJ = 5.00; Sample 2: PJ = 6.00, DCD = 1.00, ISI = 0.16, RJ = 5.00.

| Sample | fPJ | LS-FFT PJ | LS-FFT DCD | LS-FFT ISI | LS-FFT RJ | SBL PJ | SBL DCD | SBL ISI | SBL RJ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.1 MHz | 0.25 | 0.04 | 0.41 | 0.01 | 0.02 | 0.03 | 0.30 | 0.01 |
| 1 | 1.0 MHz | 1.29 | 0.04 | 0.48 | 0.06 | 0.46 | 0.03 | 0.30 | 0.01 |
| 1 | 10.0 MHz | 1.28 | 0.04 | 0.44 | 0.55 | 0.50 | 0.03 | 0.30 | 0.02 |
| 2 | 0.1 MHz | 0.44 | 0.04 | 0.42 | 0.01 | 0.01 | 0.03 | 0.30 | 0.01 |
| 2 | 1.0 MHz | 0.86 | 0.04 | 0.44 | 0.01 | 0.43 | 0.03 | 0.30 | 0.01 |
| 2 | 10.0 MHz | 0.94 | 0.04 | 0.42 | 0.20 | 0.37 | 0.03 | 0.30 | 0.02 |

As shown in Table 1, in both samples and at all three frequency points the SBL PJ error stays within 0.50 ps, while the DCD and RJ errors are bounded by 0.03 ps and 0.02 ps respectively. The LS-FFT PJ error reaches 1.28 ps (Sample 1) and 0.94 ps (Sample 2) at 10.0 MHz, and at this frequency the RJ error of Sample 1 rises to 0.55 ps, indicating that the LS-FFT frequency shift misaligns the PJ basis functions with the true signal so that the unfit PJ energy spills into the residual and biases the RJ estimate. The SBL RJ estimate stays close to its ground truth at all settings, confirming that accurate frequency identification cuts off this cross-component error propagation. Both methods over-estimate the ISI by approximately 0.30 ps regardless of the PJ frequency, which is caused by finite-sample statistical accumulation of RJ noise when the conditional means of C are taken over its 2L=32 pattern bins; this bias is intrinsic to the ISI sub-model and unrelated to the frequency identification.

To visualise the decomposition quality of SBL, the result of Sample 2 at PJ = 1 MHz is shown in Fig. 5.

![[图表/fig5_decomposition.png]]

Fig. 5 SBL decomposition result at PJ = 1 MHz.

In Fig. 5(a), the SBL-recovered PJ curve almost perfectly overlaps the ground-truth sinusoid in phase, amplitude and period, with no frequency offset or envelope mismatch — confirming that the frequency column identified by ARD from the 600-column fine dictionary corresponds to the 1 MHz ground truth and that the coefficient pair (a,b) accurately recovers the cosine and sine amplitudes. In Fig. 5(b), the residual histogram agrees closely with a zero-mean Gaussian distribution of σ=5.00 ps, with no long tail or non-Gaussian structure, indicating that the deterministic components have been fully absorbed by the dictionary sub-matrices and have not leaked into the residual.

5 Conclusion

To address the frequency-estimation bias of LS-FFT on non-uniform TIE sequences from PRBS patterns, this paper has proposed a sparse Bayesian learning approach to multi-component jitter decomposition that retains the linear superposition model of LS-FFT. A redundant frequency dictionary replaces the single-frequency column, and the ARD mechanism identifies the true PJ frequency directly from the dictionary without any frequency prior. Compared with LS-FFT, the proposed approach is no longer affected by the non-uniformity of the TIE sequence, the PJ estimation accuracy is held stable across the wide band, and the propagation of frequency-shift error to the RJ component is suppressed, increasing the reliability of jitter decomposition under PRBS non-uniform sampling. Future work will further verify the engineering applicability of the proposed method on measured oscilloscope data.

References

[1] Balestrieri, E., Picariello, F., Rapuano, S., et al.: ‘Review on jitter terminology and definitions’, Measurement, 2019, 145, pp. 264–273

[2] Chen, J., Ye, P., Zhao, Y., et al.: ‘Methodology for jitter analysis in digital storage oscilloscope’, Proc. IEEE 16th Int. Conf. Electronic Measurement & Instruments (ICEMI), Harbin, China, August 2023, pp. 1–6

[3] Tripathi, J.N., Sharma, V.K., Shrimali, H.: ‘A review on power supply induced jitter’, IEEE Trans. Compon. Packag. Manuf. Technol., 2019, 9, (3), pp. 511–524

[4] Deng, Y., Shang, Z., Tang, Z., et al.: ‘Jitter analysis and decomposition based on tail-fit algorithm’, Proc. 2024 4th Int. Conf. Communication Technology and Information Technology (ICCTIT), Guangzhou, China, December 2024, pp. 611–615

[5] Fang, S., Pan, W., Pan, D., et al.: ‘Jitter analysis based on kernel density estimation and dual-Dirac’, Proc. IEEE Int. Conf. Signal, Information and Data Processing (ICSIDP), Zhuhai, China, November 2024, pp. 1–6

[6] Peng, J., Qi, Z., Li, Z., et al.: ‘Jitter decomposition based on dual-Dirac model: a two-stage limited-sampling algorithm’, Proc. IEEE 17th Int. Conf. Electronic Measurement & Instruments (ICEMI), Beijing, China, August 2025, pp. 169–174

[7] Dou, Q., Abraham, J.A.: ‘Jitter decomposition by time lag correlation’, Proc. 7th Int. Symp. Quality Electronic Design (ISQED), San Jose, CA, USA, March 2006, pp. 525–530

[8] Yamaguchi, T.J., Ishida, M., Hou, H.X., et al.: ‘An FFT-based jitter separation method for high-frequency jitter testing with a 10× reduction in test time’, Proc. IEEE Int. Test Conf. (ITC), Santa Clara, CA, USA, October 2007, pp. 1–8

[9] Ren, N., Fu, Z., Zhou, D., et al.: ‘Jitter decomposition by convolutional neural networks’, IEEE Trans. Electromagn. Compat., 2021, 63, (4), pp. 1550–1561

[10] Duan, Y., Chen, D.: ‘Fast and accurate decomposition of deterministic jitter components in high-speed links’, IEEE Trans. Electromagn. Compat., 2019, 61, (1), pp. 217–225

[11] Tipping, M.E.: ‘Sparse Bayesian learning and the relevance vector machine’, J. Mach. Learn. Res., 2001, 1, pp. 211–244

[12] Faul, A.C., Tipping, M.E.: ‘Analysis of sparse Bayesian learning’, in Dietterich, T.G., Becker, S., Ghahramani, Z. (Eds.): ‘Advances in Neural Information Processing Systems 14’ (MIT Press, Cambridge, MA, 2002), pp. 383–389